Team Members: Mike Jones, Cole Bennet, Michael Campbell

Dr.Fareena Saqib

ECGR 3183-091

Submitted on: 12/6/2024

Computer Organization Final Project Report

# Intro

This report will go into detail about the methods used, and knowledge learned throughout the semester to be able to implement and successfully output different components needed in a basic processor. These include but are not limited to, Program counter, Register File, Instruction memory, Data memory, and the Arithmetic logical unit.

# ISA

The instruction that was given was the ability to move data from register to register, compare two registers, and basic arithmetic such as addition, and substation. Register shifting left and right, could also be implemented to form multiplication and division. Condition branching, for simplicity we implement both if zero and not.  The ability to combine or and two registers, while also having the ability to loop the instructions listed before. Else-or of two resisters was also required. Each of these required its own components and data paths for example the arithmetic logical unit required the following components, program counter, instruction memory, ALU, and a register memory unit.  The program counter manages the address of the next instruction and increments it on every clock cycle. The instruction memory unit is responsible for receiving the instruction corresponding to the PC value. The Control unit decodes the instruction and makes a control signal to operate the other components such as muxes. The control unit intakes the instruction operation code and generates a signal to guide the other components to function, needed for the ALU and the data memory. The register file contains 32 general-purpose 32-bit registers, allowing data retrieval. The ALU computes the arithmetic and logical operations of the control unit. The data memory is used for the load and store instructions or the LDUR and STUR respectively.

# Implementation

The creation of each instruction that was desired for the processor to be able to compute was constructed separately from the entire code. This was done to ensure that any mistakes were limited to syntax and not logical errors due to the new code that was made. To also aid in this process we as a group made physical diagrams of the data and control path. For simpler instructions this was not needed as there was no logical confusion, however as the instructions started to overlap with the control, the diagrams helped give us a good understanding of how and what component would be used.  To implement these functions for the processor we initially implemented an easier processor function such as simply the PC and the instructions. Once this process was completed, then it was possible to continue to layer in the more complicated instructions and functions.

# Component Designs

The components that were included are the program counter, register file unit, instruction memory, data memory, and the arithmetic logical unit. These are also basic components of any processor to function fully. Designs are as follows: (if it is not appearing please click on the drop-down menu “VHDL CODE”)

## VHDL CODE: Data Memory

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.NUMERIC_STD.ALL;

entity DataMemory is

Port ( clk : in STD_LOGIC;

address : in STD_LOGIC_VECTOR(31 downto 0);

write_data : in STD_LOGIC_VECTOR(31 downto 0);

mem_write : in STD_LOGIC;

mem_read : in STD_LOGIC;

read_data : out STD_LOGIC_VECTOR(31 downto 0));

end DataMemory;

architecture Behavioral of DataMemory is

type memory_type is array (0 to 255) of STD_LOGIC_VECTOR(31 downto 0);

signal memory : memory_type := (others => (others => '0'));

begin

read_data <= memory(to_integer(unsigned(address(7 downto 0)))) when mem_read = '1' else (others => '0');

process(clk)

begin

if rising_edge(clk) then

if mem_write = '1' then

memory(to_integer(unsigned(address(7 downto 0)))) <= write_data;

end if;

end if;

end process;

end Behavioral;

ALU

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.NUMERIC_STD.ALL;  -- Include numeric_std for arithmetic operations

entity ALU is

Port ( A : in STD_LOGIC_VECTOR(31 downto 0);

B : in STD_LOGIC_VECTOR(31 downto 0);

ALUOp : in STD_LOGIC_VECTOR(3 downto 0);

Result : out STD_LOGIC_VECTOR(31 downto 0);

Zero : out STD_LOGIC);

end ALU;

architecture Behavioral of ALU is

begin

process(A, B, ALUOp)

variable A_unsigned : unsigned(31 downto 0);

variable B_unsigned : unsigned(31 downto 0);

variable result_unsigned : unsigned(31 downto 0);

begin

-- Convert inputs to unsigned for operations

A_unsigned := unsigned(A);

B_unsigned := unsigned(B);

case ALUOp is

when "0000" =>  -- ADD

result_unsigned := A_unsigned + B_unsigned;

when "0001" =>  -- SUB

result_unsigned := A_unsigned - B_unsigned;

when "0010" =>  -- AND

result_unsigned := A_unsigned and B_unsigned;

when "0011" =>  -- OR

result_unsigned := A_unsigned or B_unsigned;

when "0100" =>  -- LSL

result_unsigned := A_unsigned sll to_integer(B_unsigned(4 downto 0));

when "0101" =>  -- LSR

result_unsigned := A_unsigned srl to_integer(B_unsigned(4 downto 0));

when others =>  -- Default

result_unsigned := (others => '0');

end case;

-- Convert result back to std_logic_vector

Result <= std_logic_vector(result_unsigned);

-- Set Zero flag using if-then-else for older VHDL versions

if result_unsigned = 0 then

Zero <= '1';

else

Zero <= '0';

end if;

end process;

end Behavioral;

Control Unit

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.NUMERIC_STD.ALL;

entity ControlUnit is

Port ( opcode : in STD_LOGIC_VECTOR(10 downto 0);

reg_write : out STD_LOGIC;

mem_write : out STD_LOGIC;

mem_read : out STD_LOGIC;

ALUSrc : out STD_LOGIC;

ALUOp : out STD_LOGIC_VECTOR(3 downto 0));

end ControlUnit;

architecture Behavioral of ControlUnit is

begin

process(opcode)

begin

case opcode is

when "10001011000" => -- ADD

reg_write <= '1'; mem_write <= '0'; mem_read <= '0'; ALUSrc <= '0'; ALUOp <= "0000";

when "11001011000" => -- SUB

reg_write <= '1'; mem_write <= '0'; mem_read <= '0'; ALUSrc <= '0'; ALUOp <= "0001";

when "11111000010" => -- LDUR

reg_write <= '1'; mem_write <= '0'; mem_read <= '1'; ALUSrc <= '1'; ALUOp <= "0000";

when "11111000000" => -- STUR

reg_write <= '0'; mem_write <= '1'; mem_read <= '0'; ALUSrc <= '1'; ALUOp <= "0000";

when others =>

reg_write <= '0'; mem_write <= '0'; mem_read <= '0'; ALUSrc <= '0'; ALUOp <= "0000";

end case;

end process;

end Behavioral;

Instruction Memory

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.NUMERIC_STD.ALL;

entity InstructionMemory is

Port ( address : in STD_LOGIC_VECTOR(31 downto 0);

instruction : out STD_LOGIC_VECTOR(31 downto 0));

end InstructionMemory;

architecture Behavioral of InstructionMemory is

type memory_type is array (0 to 255) of STD_LOGIC_VECTOR(31 downto 0);

signal memory : memory_type := (

0 => "11111000010010100111100000001010", -- Example LDUR instruction

1 => "10001011000010010101010000000101"  -- Example ADD instruction

-- Add more instructions as needed

);

begin

instruction <= memory(to_integer(unsigned(address(7 downto 0))));

end Behavioral;

PC

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

use IEEE.NUMERIC_STD.ALL;

entity PC is

Port ( clk : in STD_LOGIC;

reset : in STD_LOGIC;

PC_in : in STD_LOGIC_VECTOR(31 downto 0);

PC_out : out STD_LOGIC_VECTOR(31 downto 0));

end PC;

architecture Behavioral of PC is

signal PC_reg : STD_LOGIC_VECTOR(31 downto 0);

begin

process(clk, reset)

begin

if reset = '1' then

PC_reg <= (others => '0');

elsif rising_edge(clk) then

PC_reg <= PC_in;

end if;

end process;

PC_out <= PC_reg;

end Behavioral;

Register File

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.NUMERIC_STD.ALL;

entity RegisterFile is

Port ( clk : in STD_LOGIC;

reset : in STD_LOGIC;

read_reg1 : in STD_LOGIC_VECTOR(4 downto 0);

read_reg2 : in STD_LOGIC_VECTOR(4 downto 0);

write_reg : in STD_LOGIC_VECTOR(4 downto 0);

write_data : in STD_LOGIC_VECTOR(31 downto 0);

reg_write : in STD_LOGIC;

read_data1 : out STD_LOGIC_VECTOR(31 downto 0);

read_data2 : out STD_LOGIC_VECTOR(31 downto 0));

end RegisterFile;

architecture Behavioral of RegisterFile is

type reg_array is array (0 to 31) of STD_LOGIC_VECTOR(31 downto 0);

signal registers : reg_array := (others => (others => '0'));

begin

read_data1 <= registers(to_integer(unsigned(read_reg1)));

read_data2 <= registers(to_integer(unsigned(read_reg2)));

process(clk)

begin

if rising_edge(clk) then

if reg_write = '1' then

registers(to_integer(unsigned(write_reg))) <= write_data;

end if;

end if;

end process;

end Behavioral;

Constraints

Clock Source

set_property -dict {PACKAGE_PIN <new_pin> IOSTANDARD LVCMOS33} [get_ports clk]

create_clock -period 10.000 -name clk [get_ports clk]

Reset Pin

set_property -dict {PACKAGE_PIN V4 IOSTANDARD LVCMOS33} [get_ports reset]

# Simulation results

The following is the results of the data path and code described previously, also included are the data paths for the first part of the assignment.  These were used to test the functionality of the processor and it was being constructed to ensure that the desired values were being produced. Below is the out put of the processor, the photos that follow are the data paths used to construct the processor.